🎬

NIKON Zf + NIKKOR Z 40mm f/2

CINEMATIC MASTER GUIDE

No-ND Edition  ·  UV Filter Only  ·  India / 50 Hz

A complete field reference — camera setup, exposure doctrine, the 35-scenario master table, and on-set craft rules.

# CONTENTS

CONTENTS	2

PART 1 — SET-ONCE CAMERA SETUP	3

1.1  Physical Dials (the Zf's retro controls)	3

1.2  Video Menu	3

1.3  Display Aids	3

PART 2 — THE SEVEN CORE RULES	5

PART 3 — MASTER SCENARIO TABLE	8

PART 4 — FOCUS RULES	12

PART 5 — STABILISATION RULES	12

PART 6 — AUDIO RULES	12

PART 7 — EXPOSURE DISCIPLINE	13

PART 8 — CRAFT RULES (what actually makes it “cinematic”)	13

PART 9 — Zf-SPECIFIC GOTCHAS	13

📌  THREE DEFAULTS TO MEMORISE	15

⚡  SIX RULES THAT GOVERN EVERYTHING ABOVE	15

# PART 1 — SET-ONCE CAMERA SETUP

Do this today, then never think about it again.

## 1.1  Physical Dials (the Zf's retro controls)

| Dial | Set to | Why |
| --- | --- | --- |
| ISO dial | C | Frees ISO to the command dial — essential for video |
| Shutter speed dial | 1/3 STEP | Frees shutter to the command dial |
| Mode dial | M | Manual is the only cinematic exposure mode |
| Photo / Video / B&W switch | Video | — |
| Front sub-dial | Aperture | — |
| Rear command dial | Shutter | — |

## 1.2  Video Menu

| Menu item | Set to | Why |
| --- | --- | --- |
| Movie file type | H.265 10-bit (MOV) | 10-bit = far more colour data for grading |
| Frame size / rate | 3840×2160 25p | Oversampled from 6K, full sensor width, sharpest |
| Tone mode | SDR + Flat Picture Control (daily) · N-Log (controlled/indoor/night) | Log's ISO 800 base costs 3 stops — painful without ND |
| Flat PC tweak | Contrast −2 · Saturation −1 · Sharpening +1 | Grade-friendly but fast to finish |
| Electronic VR | OFF (ON only for walking shots) | e-VR adds ~1.1× crop |
| Flicker reduction | 50 Hz | India mains. Kills LED/tube banding |
| Separate exposure & WB for video | ON | Stops stills settings bleeding in |
| Movie AF speed | −5 | Slow, cinematic focus pulls |
| AF tracking sensitivity | 4–5 (slow) | Stops hunting when someone crosses frame |
| Mic sensitivity | Manual, peaks −12 dB | Auto gain pumps and hisses |
| Attenuator / Wind reduction | Situational | — |
| Card | UHS-II V60/V90 SD in slot 1 | microSD slot is too slow for 4K |

## 1.3  Display Aids

| Tool | Setting | Assign to |
| --- | --- | --- |
| Zebra pattern | ON, 95% | — |
| Peaking highlight | ON, red, level 2–3 | — |
| Waveform | ON | Fn1 |
| N-Log View Assist | ON when in Log | Fn2 |
| Punch-in / magnify | — | Record-button-hold or i-menu |

# PART 2 — THE SEVEN CORE RULES

### ① Frame Rate — 25p is your default

- 25p = normal cinematic motion, flicker-free under Indian lights

- 50p = 2× slow motion (conform to 25p in edit)

- 1080/100p or 120p = 4–5× slow motion, hero shots only

⚠️ 4K 50p/60p forces a 1.5× DX crop → your 40mm becomes a 60mm equivalent. Never mix 24p and 25p in one project.

### ② Shutter — 180° is the goal, not a law

Shutter = 1 ÷ (2 × fps)

| fps | Ideal shutter |
| --- | --- |
| 24p | 1/50 |
| 25p | 1/50 |
| 50p | 1/100 |
| 100p | 1/200 |

Break it upward in harsh sun (no ND). Break it downward (1/25) only at night, on a tripod.

### ③ ISO — live at base

| Tone mode | Base | Ceiling |
| --- | --- | --- |
| Flat SDR | 100 | 6400 (12800 in emergency) |
| N-Log | 800 (never below) | 6400 |

Never chase exposure with ISO. Fix light → fix aperture → fix shutter → then ISO.

### ④ Aperture — doing double duty (look and exposure)

- f/2 — gorgeous separation, brutal DOF. Static shots. Night default.

- f/2.8–f/4 — sweet spot for talking heads and moving subjects

- f/5.6–f/8 — daylight workhorse, groups, establishing

- f/11–f/16 — only to hold 1/50 in strong light. Accept diffraction.

f/16 is the hard ceiling. f/2 is the floor.

Depth of field at 40mm (full frame):

| Distance | f/2 | f/4 | f/8 |
| --- | --- | --- | --- |
| 1.5 m (head & shoulders) | 17 cm | 35 cm | 72 cm |
| 2 m (waist-up) | 30 cm | 61 cm | 1.3 m |
| 3 m (full body) | 69 cm | 1.4 m | 3.1 m |
| 5 m | 2.2 m | 5 m | ~∞ |

### ⑤ White Balance — always manual Kelvin, never AWB

| Situation | Kelvin |
| --- | --- |
| Midday sun | 5500–5600 K |
| Golden hour (keep it warm) | 6500–7500 K |
| Shade / overcast | 6500 K |
| Blue hour | 4200–4500 K |
| Tungsten / warm indoor | 3200 K |
| Mixed LED interior | Grey-card custom |
| Neon / night street | 3400–3800 K |
| Candle / diya | 2800–3200 K |

### ⑥ The No-ND Exposure Doctrine

Without ND, light management replaces filtration. Order of operations in bright light:

- Change when — golden hour, blue hour, overcast = free ND

- Change where — open shade is 3–4 stops down, and rescues 180° instantly

- Stop down — keeps motion blur, loses bokeh

- Raise shutter — keeps bokeh, loses motion blur

- Shoot 50p and slow it in post — judder disappears. Best no-ND hack there is.

Where 180° survives (25p, 1/50, ISO 100):

| Light | Aperture at 1/50 |
| --- | --- |
| Dim interior | f/2 |
| Average interior | f/2.8 |
| Bright interior, big window | f/4 |
| Deep shade / dusk | f/5.6 |
| Open shade / heavy overcast | f/8 |
| Overcast | f/11 |
| Bright overcast / hazy sun | f/16 ← the limit |
| Direct bright sun | Impossible — 1–1.5 stops past f/16 |

Direct sun ladder (ISO 100) — pick a rung:

| Aperture | Shutter | Trade-off |
| --- | --- | --- |
| f/2.8 | 1/3200 | Max bokeh, staccato motion — locked off only |
| f/4 | 1/1600 | Nice separation, crisp motion |
| f/5.6 | 1/1000 | Balanced |
| f/8 | 1/500 | Best all-round compromise |
| f/11 | 1/250 | Deep focus, near-natural motion |
| f/16 | 1/125 | Closest to 180°, softest (diffraction) |

While high-shuttering: lock off or move very slowly, avoid fast walk-bys, prefer 50p→slow-mo, and add a touch of motion blur in post if needed.

### ⑦ Flicker Law (India)

Only 1/50 and 1/100 under artificial light. 1/60 and 1/125 will band under Indian tube lights and cheap LEDs. Always do a 5-second flicker test roll before an event.

# PART 3 — MASTER SCENARIO TABLE

Constants for every row: 4K UHD (unless stated) · MOV H.265 10-bit · M exposure · Manual Kelvin WB · Flicker Reduction 50 Hz · AF-C · AF speed −5 · Tracking sensitivity 4–5 · ISO dial = C · Shutter dial = 1/3 STEP · Zebra 95% · Waveform on Fn1

| # | Scenario | fps | Shutter | Aperture | ISO | WB | Tone | Stabilisation / Focus | Zf Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ☀️ DAYLIGHT — OUTDOOR |  |  |  |  |  |  |  |  |  |
| 1 | Outdoor interview, open shade | 25p | 1/50 | f/4–5.6 | 100 | 6500 K | Flat SDR | IBIS ON · e-VR OFF · Wide-Area (L-people) | Best-case no-ND setup. True 180° holds. |
| 2 | Outdoor interview, direct sun | 25p | 1/500 | f/8 | 100 | 5600 K | Flat SDR | Tripod / IBIS OFF · Wide-Area (L-people) | Judder — lock off. Better: move them into shade. |
| 3 | Golden hour portrait | 25p | 1/50 | f/2.8–4 | 100 | 7000 K | Flat SDR | IBIS ON · e-VR OFF · Eye-detect AF-C | Backlight + white bounce. Golden hour = your ND. |
| 4 | Midday b-roll, open ground | 25p | 1/500 | f/8 | 100 | 5600 K | Flat SDR | IBIS ON · e-VR OFF · MF at hyperfocal | Locked-off or very slow moves only. |
| 5 | Midday but you want bokeh | 50p→25p | 1/1000 | f/2.8 | 100 | 5600 K | Flat SDR | IBIS ON · e-VR OFF · AF-C tracking | Conform to 25p slow-mo — judder vanishes. |
| 6 | Overcast street | 25p | 1/50 | f/8–11 | 100 | 6500 K | Flat SDR | IBIS ON · e-VR OFF · AF-C Auto-area | The gift day. 180° with zero compromise. |
| 7 | Street walking shot (day) | 25p | 1/50 | f/4 | 400–1600 | 5600 K | Flat SDR | IBIS ON · e-VR ON · AF-C tracking | Accept 1.1× crop. Heel-toe walk, knees bent. |
| 8 | Landscape / establishing | 25p | 1/50 | f/8–11 | 100 | 5600 K | Flat SDR | Tripod · IBIS OFF · MF + punch-in | IBIS drifts when locked off. |
| 9 | Rain / monsoon street | 25p | 1/50 | f/4–5.6 | 400–1600 | 6000 K | Flat SDR | IBIS ON · e-VR OFF · AF-C tracking | Wet ground doubles your light. Shield micro-HDMI port. |
| 🏠 INDOOR — DAYTIME |  |  |  |  |  |  |  |  |  |
| 10 | Indoor interview, window light | 25p | 1/50 | f/2.8 | 400–1600 | 4000 K | N-Log | Tripod · IBIS OFF · Eye-detect, then lock | Window at 45°. Negative fill on shadow side. |
| 11 | Café / restaurant b-roll | 25p | 1/50 | f/2 | 800–3200 | 3800 K | N-Log | IBIS ON · e-VR OFF · MF + peaking | Elbows braced. Grey-card WB — mixed LEDs lie. |
| 12 | Product / food macro | 25p | 1/50 | f/4–5.6 | 100–400 | 5000 K | Flat SDR | Tripod · IBIS OFF · MF + punch-in | Min focus 0.29 m, 0.17× max. Never trust AF this close. |
| 13 | Office / corporate talking head | 25p | 1/50 | f/2.8–4 | 400–1600 | 4300 K | N-Log | Tripod · IBIS OFF · Wide-Area (L-people) | Tube-light banding → 1/50 only, never 1/60. |
| 🌆 BLUE HOUR (the best 20 minutes) |  |  |  |  |  |  |  |  |  |
| 14 | Blue hour cityscape / skyline | 25p | 1/50 | f/4–5.6 | 400–1600 | 4500 K | N-Log | Tripod · IBIS OFF · MF at infinity, punch-in | 🏆 Best light of the day. Window is 20–25 min — shoot fast. |
| 15 | Blue hour portrait / walk | 25p | 1/50 | f/2–2.8 | 800–3200 | 4200 K | N-Log | IBIS ON · e-VR OFF · Eye-detect AF-C | Sky as backdrop, practical light as key. |
| 🌙 NIGHT — STREET & CITY |  |  |  |  |  |  |  |  |  |
| 16 | Neon street / market b-roll | 25p | 1/50 | f/2 | 3200–6400 | 3600 K | Flat SDR | IBIS ON · e-VR OFF · AF-C tracking (slow) | Expose for the sign, not the street. Let blacks crush. |
| 17 | Night portrait under streetlamp | 25p | 1/50 | f/2 | 3200–6400 | 3400 K | Flat SDR | IBIS ON · MF locked (phone-torch pre-focus) | Lamp behind/45° above = rim light. DOF at 1.5 m is only 17 cm. |
| 18 | Night street walking / POV | 25p | 1/50 | f/2–2.8 | 6400 | 3800 K | Flat SDR | IBIS ON · e-VR ON · AF-C tracking | f/2.8 buys focus safety — worth one stop of ISO. |
| 19 | Dim lane / single distant lamp | 25p | 1/50 | f/2 | 6400–12800 | 3200 K | Flat SDR | IBIS ON · brace on wall · MF locked | Denoise mandatory. Deliver 1080p from 4K. |
| 20 | Night traffic / light trails | 25p | 1/25 | f/2.8–4 | 800–1600 | 3600 K | N-Log | Tripod · IBIS OFF · MF locked | 360° shutter = +1 stop + beautiful smear. Tripod only. |
| 🌙 NIGHT — INTERIORS & EVENTS |  |  |  |  |  |  |  |  |  |
| 21 | Bar / lounge / café (night) | 25p | 1/50 | f/2 | 1600–3200 | 3200 K | N-Log | IBIS ON · e-VR OFF · MF + peaking L3 | Seat subject beside a practical. ETTR +1 stop. |
| 22 | Night interview, one LED light | 25p | 1/50 | f/2.8 | 800–1600 | 3200 K | N-Log | Tripod · IBIS OFF · Eye-detect, then lock | Key at 45°, negative fill. Indian skin 42–50 IRE. |
| 23 | House party / home function | 25p | 1/50 | f/2 | 3200–6400 | 3600 K | Flat SDR | IBIS ON · e-VR OFF · AF-C People | Do a 5 s flicker test roll first. 1/50 only. |
| 24 | Wedding reception / stage lights | 25p | 1/50 | f/2–2.8 | 1600–6400 | 4200 K | Flat SDR | IBIS ON · e-VR OFF · AF-C tracking | ⚠️ Coloured LED wash clips RED first — underexpose faces ⅓ stop. |
| 25 | Concert / live stage | 25p | 1/50 | f/2 | 3200–12800 | 4000 K | Flat SDR | IBIS ON · brace · AF-C, Auto ISO cap 6400 | Meter the brightest moment, not the darkest. |
| 26 | Diwali diyas / candlelight face | 25p | 1/50 (or 1/25) | f/2 | 6400–12800 | 2800–3200 K | Flat SDR | IBIS ON (OFF + tripod if 1/25) · MF locked | Let flames clip. Hidden warm LED at 5–10% for shadow side. |
| 27 | Fireworks / festival lights | 25p | 1/50 | f/2.8–4 | 800–3200 | 3800 K | N-Log | Tripod · IBIS OFF · MF at infinity | Bursts are bright — stop down. Expose just under zebra. |
| 🚗 VEHICLE & MOVEMENT |  |  |  |  |  |  |  |  |  |
| 28 | Car interior, parked (night) | 25p | 1/50 | f/2 | 6400 | 3400 K | Flat SDR | IBIS ON · e-VR OFF · MF locked | Dash glow + phone bounce = free key. Kill AC vent noise. |
| 29 | Moving vehicle / dash POV | 25p | 1/50 | f/2 | 6400 | 3600 K | Flat SDR | Clamped mount · IBIS ON · MF locked | ⚠️ Rolling shutter skews passing poles. Steady the camera, not the car. |
| 30 | Gimbal move (any light) | 25p | 1/50 day-shade / 1/500 sun | f/8 day · f/2 night | as needed | as scene | match scene | Gimbal · IBIS ON · e-VR OFF · AF-C slow | 40mm is tiny — easy load. Slow, deliberate moves only. |
| ⚡ SLOW MOTION |  |  |  |  |  |  |  |  |  |
| 31 | Slow-mo hero shot (day) | 1080/100p | 1/200 | f/2.8–5.6 | 400+ | match | Flat SDR | IBIS ON · e-VR OFF · MF pre-set | Or 4K 50p — but remember the 1.5× DX crop (40 → 60 mm). |
| 32 | Slow-mo (night, lit subject) | 1080/50p | 1/100 | f/2 | 6400 | 3600 K | Flat SDR | Tripod · IBIS OFF · MF locked | Costs 1 stop. Never 100/120p at night — noise disaster. |
| 33 | 2× cinematic slow-mo (general) | 50p→25p | 1/100 | as scene | as scene | match | match scene | IBIS ON · e-VR OFF · AF-C slow | The universal “looks expensive” hack. |
| 🌌 SPECIAL |  |  |  |  |  |  |  |  |  |
| 34 | B&W night street | 25p | 1/50 | f/2 | 6400 | n/a | Monochrome / Flat Mono | IBIS ON · MF zone-focused at 2 m | ⚠️ Colour unrecoverable — commit deliberately. Noise reads as grain. |
| 35 | Astro / star field | — | — | f/2 | 3200–6400 | 4000 K | Photo + interval timer | Tripod · IBIS OFF · MF at infinity via punch-in | Real-time video not viable at 40 mm. Shoot stills → timelapse. |

# PART 4 — FOCUS RULES

- AF-C + Subject detection (People/Auto); AF area = Wide-Area (L-people) for interviews, Subject-tracking for movement

- AF speed −5, tracking sensitivity 4–5 — this is what makes AF look like a focus puller, not a webcam

- Manual focus for anything critical: peaking + punch-in to confirm, then lock. The control ring is fly-by-wire — turn slowly

- Focus first, then roll. Never hunt on camera

- Tape-mark floor positions for repeatable rack focus

- Minimum focus = 0.29 m. Closer and you'll only hear the motor

- Night = manual focus by default. Pre-focus with a phone torch, then lock

- Silver lining of no ND: daylight often puts you at f/5.6–f/8, where focus is far more forgiving

# PART 5 — STABILISATION RULES

| Shot type | IBIS | e-VR | Notes |
| --- | --- | --- | --- |
| Tripod / locked-off | OFF | OFF | IBIS drifts on a tripod |
| Handheld static | ON | OFF | Two hands, elbows in, exhale, hold 8–10 s |
| Handheld walking | ON | ON | Accept 1.1× crop; bend your knees |
| Gimbal | ON | OFF | 40mm is tiny — easy on any small gimbal |
| Pan / tilt | ON | OFF | A 90° pan should take ≥ 6 seconds at 40mm |
| Vehicle mount | ON | OFF | Clamp hard; rolling shutter is the real enemy |

40mm is not wide — shake shows more than on a 20mm. Brace against a wall or pole. With a high daylight shutter, slow moves are mandatory, not optional.

# PART 6 — AUDIO RULES

- Internal mic = sync/reference only. Always plan a real mic

- 3.5 mm input → shotgun (on-camera) or lav/wireless (best for dialogue)

- Manual level, dialogue peaking −12 dB, never touching 0

- Attenuator ON in loud environments (concerts, weddings, markets)

- Wind muff outdoors, always

- Record 10 s of room tone at every location

- Kill AC vents, fans and fridges before rolling indoors

# PART 7 — EXPOSURE DISCIPLINE

### Flat SDR (your daylight and night-street mode)

- Zebras at 95% — highlights should just begin to trigger

- Skin tones ≈ 60–70 IRE on the waveform

- Light grade in post — done in minutes

### N-Log (interiors, blue hour, night with control)

- Base ISO 800 minimum

- ETTR by 1–1.5 stops — underexposed Log = noisy, muddy shadows

- Light skin ≈ 50–55 IRE · Indian / medium skin ≈ 42–50 IRE

- View Assist ON so the monitor doesn't lie

- Grade from Nikon's official N-Log → Rec.709 LUT, then adjust

- Don't shoot Log outdoors without ND — you start 3 stops in the hole for no benefit

### The two golden habits

- Day: protect highlights. Blown skies and clipped skin can never be recovered.

- Night: protect the face, let the blacks crush. Grey mush is worse than deep black.

# PART 8 — CRAFT RULES (what actually makes it “cinematic”)

- Hold every shot 10 seconds minimum, even if you use 3. Editors need handles

- One movement per shot. Pan or push in — never both

- Motivate movement — the camera moves because the subject moves

- Shoot coverage sets: wide → medium → close → detail. 40mm gives medium naturally; physically move for close-ups

- Backlight + negative fill beats front light every time — and drops exposure into 180°-friendly territory

- Golden hour is your ND. Schedule around it religiously

- Foreground layers — shoot through leaves, doorways, glass. Instant depth at f/2

- Eye-level or below for heroism; above for vulnerability

- Lock fps + WB across the shoot; keep shutter/aperture consistent within each scene

- Overcast days are a gift — f/5.6 at 1/50 with zero compromise

# PART 9 — Zf-SPECIFIC GOTCHAS

| Gotcha | Mitigation |
| --- | --- |
| ~125 min clip limit | Split long interviews anyway |
| Micro HDMI (Type D) — fragile | Cage/clamp if monitoring externally; shield in rain |
| microSD slot too slow for 4K | Keep everything on the full-size UHS-II SD |
| Rolling shutter | Avoid whip pans and hard vertical lines during fast moves |
| Battery EN-EL15c | Carry 3+; USB-C PD power bank for static shoots |
| 40mm has no OIS | You rely entirely on IBIS — keep it on handheld |
| Mild focus breathing | Keep pulls short, or hide them under motion |
| Lens min aperture f/16 | The hard ceiling of your no-ND daylight range |
| 4K 50/60p = 1.5× DX crop | 40mm → 60mm equivalent. Don't intercut carelessly |
| e-VR = 1.1× crop | Frame wider before turning it on |

# 📌  THREE DEFAULTS TO MEMORISE

| Condition | Setting |
| --- | --- |
| 🌤️ Normal light (shade, indoor, overcast, golden hour) | 25p · 1/50 · ISO 100 · f/2.8–5.6 · Flat SDR · WB by chart · IBIS ON |
| ☀️ Harsh direct sun | 25p · 1/500 · ISO 100 · f/8 · 5600 K — slow moves only; or 50p · 1/1000 · f/2.8–4 → conform to 25p slow-mo |
| 🌙 Night / low light | 25p · 1/50 · f/2 · ISO 3200–6400 · 3400–3800 K · Flat SDR · IBIS ON. Controlled/tripod: 1/50 or 1/25 · f/2 · ISO 800–1600 · N-Log · ETTR +1 stop |

# ⚡  SIX RULES THAT GOVERN EVERYTHING ABOVE

- Shutter = 1 ÷ (2 × fps). Break it upward in sun; downward (1/25) only at night on a tripod.

- Base ISO first — 100 for SDR/Flat, 800 for N-Log. Never chase exposure with ISO.

- f/16 is the hard ceiling in daylight; f/2 is the floor at night.

- Only 1/50 and 1/100 under Indian artificial light — 1/60 and 1/125 will band.

- Day: protect highlights. Night: protect the face, let the blacks crush.

- Consistency within a scene beats correctness across scenes — never cut f/8 @ 1/500 against f/2.8 @ 1/50.

Deviate deliberately, never accidentally.